MAGNETIC PROPERTIES OF SOILS AND SOIL CROSS-SECTION INFLUENCE ON ANOMALOUS MAGNETIC FIELD STRUCTURE

Lyudmila A. Zolotaya, Maria V. Kalisheva, Sergei A. Tikhotsky

Moscow State University, Geological Faculty, Dept. of Geophysics, 119899 Moscow, Russia

E-mail: masha@geophys.geol.msu.ru

INTRODUCTION

Investigations of soils along with their components and constituent minerals demonstrated, that soils of any kind have certain magnetic properties due to obvious paramagnetism of principal constituent minerals on one hand and due to certain content of ferromagnetic iron compounds, e. g. magnetite, pyrrotine, hematite, etc.

Magnetometry is lately involved actively into studying shallow-depth (up to 1.5 m) object of civil engineering and archaeology with low magnetic activity. The authors’ experience in this industry resulted in posing the problem of considering the soil cross-section magnetic influence on the anomalous magnetic fields.

The following program was implemented for solving this problem: 1) measurement of magnetic susceptibility at monolith soil samples; 2) statistical analysis of the obtained data; 3) modelling the magnetic field of the contact of soils with different magnetic susceptibility cross-sections; 4) modelling the magnetic field of soil surface microrelief with various magnetic activity of the soil layer.

SOIL SAMPLES MAGNETIC SUSCEPTIBILITY MEASUREMENTS

132 soil monolith samples from collections of Moscow State University Soil Faculty and Moscow State University Earth Studies Museum were investigated. Different soil varieties are presented in these collections - black soil (26 per cent of the surveyed samples), saline soils (16 per cent), brown soils (11 per cent), podzols (20 per cent), gray soils (19 per cent), yellow and red soils (8 per cent). Soil samples gathered in various regions of Russia and the Ukraine; samples of certain soil type represent the whole area of this type.

1692 magnetic susceptibility measurements were made on well-preserved samples with KT-5 (GEOFYZIKA BRNO, Czechoslovakia) equipment, providing the single measurement accuracy of 10-5 SI units of magnetic susceptibility. The sensor was moved along the sample’s surface; measurements were taken at interval of 6 cm up to the depth of 80-120 cm.

Fig. 1.Magnetic susceptibility histograms for black soil (a), podzol (b), brown soil (c), saline soil (d), gray soil (e), red soil (f).

STATISTICAL ANALYSIS

Fig. 2. Magnetic profiles of different soil varieties

Statistical approach was applied to analyze the obtained magnetic susceptibility data. The notion of statistical distribution is generally the basis of such an approach. It seems that no systematic investigation of soil magnetic susceptibility distribution was ever undertaken.

Lognormal distribution (normal distribution of logarithms) hypothesis for magnetic susceptibility within any of the investigated soil varieties was verified using Kolmogorov-Smirnov criterion. For all soil varieties the lognormal distribution hypothesis is approved with the significance level of 10-20 per cent. All magnetic susceptibility values for a certain soil variety were used for verification regardless to the corresponding depth. The magnetic reseptivity histograms, thus derived, for each soil variety (Fig. 1) represent the average magnetic susceptibility value for the depth interval as a whole.

Black, brown and gray soil cannot be distinguished by magnetic susceptibility average values. The podzols and saline soils have the least values of average magnetic susceptibility along with the least values of magnetic susceptibility variance, while the red and yellow soils have greatest variance value. The soil varieties can thus be ordered from those with the least magnetic susceptibility to those with the greatest as follows: podzols-saline soils-brown-gray-black-yellow and red soils. Two groups of soil types can be distinguished by their magnetic susceptibility: 1) the group of low-magnetic soils, including podzols, saline, brown, gray and black soils with magnetic susceptibility varying from 5·10-5 SI units to 80·10-5 SI units; and 2) the group of high-magnetic soils, including yellow and red soils with magnetic susceptibility exceeding 160·10-5 SI units.

The parameters of  the lognormal distribution are changed with depth. Magnetic susceptibility histograms for various depths are presented at. Fig. 2. Soil magnetic profile representing the depth dependency of magnetic susceptibility can be thus obtained for each soil variety along with variance limits. Magnetic profile seems to an important feature of the soil variety.

Magnetic profiles of  black soils and brown soils are the most contrast as certain soil horizons are presented by distinct extremums. Magnetic susceptibility, being of  (5-80)·10-5 SI units at depths less than 20 cm, decreases sharply to the half of that value in the depth interval from 40 to 50 cm, and further on varies slightly within the limits of 5·10-5 SI units to 20·10-5 SI units. Sharp change of the magnetic susceptibility seems to correspond the transient layer (so called layer B) from soil (layer A) to subsoil or bedrock (layer C). Magnetic profiles for other soil varieties are less expressive, which may result from less degree of the bedrock alteration in the course of soil genesis.

Fig.3. The model of podzol - black soil contact

Magnetic profiles of saline soils and gray soils are almost constant at all depth values at the level of (5-20)·10-5 SI units and magnetic susceptibility variance remains constant as well.

Magnetic profiles for podozols differs significantly from other soil varieties as magnetic susceptibility increases slightly with the depth, while the variance is the greatest in the depth interval from 30 to 50 cm.

THE CONTACT OF SOILS WITH DIFFERENT MAGNETIC PROFILES

Contacts of different soil varieties were modelled, keeping the notion of real soil contacts in mind. The model of podzol - black soil contact, typical for the Central Russia is presented at Fig. 3. The values of magnetic susceptibility of the cells at certain depths are those of corresponding soil variety at the corresponding depth according to this variety’s magnetic profile derived on the previous stage of investigation. The amplitude of the anomaly at the contact makes 7 nTl at the soil’s surface (zero depth value). It seems that, with a certain field technique and equipment, providing the survey accuracy of 1 nTl, the contact of soils can be thus mapped.

SOIL LAYER INHOMOGENEITY EFFECT ON MAGNETIC FIELD STRUCTURE

The properties and geometry of soil layer are controlled by a number of factors. The surface’s microrelief  is one of significant ones. A model of small (20 cm deep) trench in black soil is presented at Fig. 4a. The trench is expressed in 10 nTl negative anomaly in the magnetic field at the surface. If a magnetoactive body is present in the soil (Fig. 4b), the anomalous field becomes complicated. Interpreting such anomalies formally is likely to result in distinguishing false objects into the interpretational model. This can be avoided by introducing exact surface altitudes into the interpretational model. Precise magnetic survey for the purposes of civil engineering or archaeology should thus be always accompanied by precise altitude measurements at the survey site.

CONCLUSIONS

- Like those of the sedimentary rocks, the values of magnetic susceptibility of soils are submitted to the lognormal law of distribution.

- Considering average magnetic susceptibility, two groups of soil types can be distinguished:

low-magnetic soils, including podzols, saline, brown, gray and black soils with magnetic susceptibility varying from 5·10-5 SI units to 80·10-5 SI units;

Fig. 4. A model of small trench in black soil (a) and joint effect of the trench and the magnetoactive body (b)

- high-magnetic soils, including yellow and red soils with magnetic susceptibility exceeding 160·10-5 SI units.

- Magnetic profiles, i. e. depth dependencies of magnetic susceptibility and its distribution parameters, differ for different soil varieties, which can be thus mapped.

- According to the modelling results, it is possible to map the contacts of soil varieties by means of precise magnetometry.

- Soil surface microrelief effect on the magnetic field structure should be taken into the account when interpreting precise magnetometry data.