EQUIVALENT DIPOLES APPROACH TO THE ELECTRICAL FIELD VECTOR MEASUREMENTS DATA INTERPRETING

Aleksandr A. Gorbunov, Igor. N. Modin

Moscow State University, Geological Faculty, Dept. of Geophysics, 119899 Moscow, Russia

E-mail: gorb@geophys.geol.msu.ru

INTRODUCTION

Estimating construction conditions is an important problem facing those engaged in construction protection against corrosion to prevent their destruction which is likely to result in severe economical losses and environmental damage. The constructions, contacting the ground or buried under the ground surface, reveal themselves as anomalous objects in the layered medium or on its surface. If the metal of the construction is in the galvanic contact with the embedding rock, the object appears to be highly conductive. Concrete or covered with water-proof coating constructions appear as the objects of high resistivity. Thus the ability appears to use resistivity techniques for estimating the conditions of the constructions.

Commonly used techniques of resistivity prospecting suppose that the target object is crossed by the observation line. Sometimes it is impossible to apply such techniques as there is no direct access to the place. So a technique is needed capable of investigating the objects aside the observation line.

Besides that, traditional techniques involve, generally, only one component of the electric field, precisely, that in the observation line direction. Using two orthogonal components, i. e. taking the vector properties of the electric field into consideration, is likely to provide additional information concerning 3D inhomogenities in the layered cross-section.

ELECTRICAL FIELD VECTOR MEASUREMENTS (EFVM) TECHNIQUE

The EFVM technique (Fig.1) was introduced in 1992 by the MSU team (Modin, Shevnin et al., 1992; Modin, Ignatova, 1995), implementing the above mentioned approach. Two values of the potential difference Ux and Uy are measured with orthogonal positions of the measurement line MN are measured at a number of points O (center of MN) along the observation line encircling the supposed location of the target object. The measurement are implemented at the same points with several locations of the single current electrode A introducing the electric current I into the cross-section. The current electrode locations surround the object location as well. The second current electrode B is placed in the «infinity». The measurements are implemented taking the sign of the potential difference value into consideration.

Generalizing the traditional apparent resistivity ρa=EMNj0=2p⋅AO2MN⋅DUMNI for the pole-dipole (half-Schlumberger) array, the apparent resistivity vector ρa=Ej0 with components ρax=Exj0=2p⋅AO2MN⋅DUxI,ρay=Eyj0=2p⋅AO2MN⋅DUyI is introduced.

The field theory says, that the electrical field in the inhomogeneious cross-section can be presented as the sum of the normal field and anomalous field. The normal field is the field of the current source in the embedding cross-section without inhomogenities. The anomalous field is the field of the secondary sources induced by the normal field at the boundaries of the inhomogeneities. Considering the layered cross-section as the embedding medium, apparent resistivity vector can be presented as ρa=ρa0+ρaan - the sum of the normal and the anomalous apparent resistivity. The value of ρa0 is the traditional apparent resistivity for the embedding layered cross-section, and its direction is the direction of the AO radius-vector. Thus the anomalous apparent resistivity vector ρaan=ρa−ρa0 can be estimated, representing the effect of the inhomogeneity as if in the semi-space. The ρaan vector is oriented in the same way as the anomalous electric field and brings the information concerning the inhomogeneity.

EQUIVALENT DIPOLES APPROACH TO EFVM DATA INTERPRETING

The anomalous field of the secondary sources representing the isolated inhomogeneity in the primary field of the point current source can be approximated with the field of dipole or so called ‘real dipole’ - a system of two point sources of opposite signs separated by certain distance. The properties of this dipole - its position in the cross-section and the dipole moment vector value and direction - are connected with the properties of the corresponding object. The EFVM inverse problem can be thus posed as follows: to derive the properties of the dipole in the semi-space from the observed vector of the dipole’s electric field on the surface of the semi-space (equivalent dipoles approach).

Wide-spread approach of minimizing the fitting error between the measured and calculated electric field values by varying, within certain limits, the dipole’s properties is applied. Various dipole’s properties sets, though physically equivalent, are different regarding the calculational effectiveness of the minimizing process. These sets are: dipoles’ coordinates and dipole moment vector components; ‘real dipole’s’ coordinates, intensity, length, dip angle and dip direction; poles’ coordinates and intensities separately. The latest approach proved to be the most effective one. Solving the inverse problem for the data gained with several current electrode location, a set of dipoles is obtained. The spatial area occupied by this set marks the boundaries of the anomalous object (Fig. 2).

REFERENCES

- Modin I. N., Shevnin V. A. et al. Vector measurements in resistivity prospecting. EAEG 56th Ann. Meeting, Vienna, Austria, 1994.

- Modin I. N., Ignatova I. D. Coal layer inhomogeneities investigations by vector resistivity measurements in mines. EAGE 57th Ann. Meeting, Glasgo, UK, 1995.

- Fig. 1.The EFVM technique: current electrode (1), -	Fig. 2. EFVM data intrepreting using equivalent dipoles

- positive (2) and negative (3) secondary sources,	approach: current electrode (1) and positive (2) and

- primary (4), secondary (5) and total (6) field,	negative (3) poles of the equivalent dipoles, obtained

- measured field components (7).	for different current electrodes position.